PATENT

IN THE UNITED STATES PATENT AND TRADEMARK OFFICE

| In re Application of: ZHOU, Jingjie et al. Application No.: To Be Assigned (Continuation of International Application No. PCT/CN2020/079488) Filed: On even date herewith For: SYSTEMS AND METHODS FOR GENERATING ADAPTIVE RADIATION THERAPY PLAN |  | Group Art Unit: To Be Assigned Examiner: To Be Assigned Confirmation No.: To Be Assigned |
| --- | --- | --- |

Commissioner for Patents

P.O. Box 1450

Alexandria, VA 22313-1450

Dear Commissioner:

PRELIMINARY AMENDMENT

Prior to the examination of the above application, please amend this application as follows:

Amendments to the Claims begin on page 2 of this paper.

Remarks begin on page 16 of this paper.

CLAIM AMENDMENTS

Please amend the claims as indicated below.  This listing of claims will replace all prior versions and listings of claims in the application:

1. (Original) A method for generating an adaptive radiation therapy plan, implemented on a machine having one or more processors and one or more storage devices, the method comprising:

obtaining a first image related to one or more target objects generated by a first scan;

obtaining a first radiation therapy plan for treating the one or more target objects, the first radiation therapy plan being associated with the first image;

obtaining a second image related to the one or more target objects generated by a second scan, the second scan being performed later than the first scan; and

determining, based on the first radiation therapy plan, the first image, and the second image, a target radiation therapy plan to treat the one or more target objects, the target radiation therapy plan being the first radiation therapy plan or a second radiation therapy plan associated with the second image, wherein at least a portion of the determining the target radiation therapy plan are performed in parallel.

2. (Original) The method of claim 1, wherein the determining the target radiation therapy plan includes:

generating a registration result by performing image registration between the first image and the second image;

determining one or more regions of interest (ROIs) in the second image based on the registration result;

determining an accumulated dose in the second image based on the registration result, the accumulated dose having been delivered to the one or more target objects; and

estimating a first dose distribution in the second image based on the first radiation therapy plan, wherein

an operation for determining the one or more ROIs in the second image based on the registration result and an operation for determining the accumulated dose in the second image based on the registration result are performed in parallel; or

an operation for estimating the first dose distribution in the second image based on the first radiation therapy plan is performed in parallel with at least one of an operation for generating the registration result by performing the image registration between the first image and the second image and the operation for determining the one or more ROIs in the second image based on the registration result.

3. (Original) The method of claim 2, wherein the determining the one or more ROIs in the second image includes:

automatically determining one or more first ROIs of the one or more ROIs in the second image;

receiving a first instruction for modifying or accepting the one or more first ROIs that are automatically determined;

processing at least one of the one or more first ROIs based on the first instruction; and

automatically determining one or more second ROIs of the one or more ROIs in the second image,

wherein an operation for automatically determining the one or more second ROIs of the one or more ROIs in the second image is performed in parallel with at least one of an operation for receiving the first instruction for modifying or accepting the one or more first ROIs that are automatically determined and an operation for processing the at least one of the one or more first ROIs based on the first instruction.

4. (Currently Amended) The method of claim 2[[ or 3]], wherein the determining the one or more ROIs in the second image includes:

estimating a length of time for determining each of the one or more ROIs;

ranking the lengths of time; and

automatically determining the one or more ROIs based on the ranking result.

5. (Currently Amended) The method of claim 2any one of claims 2-4, wherein the determining the one or more ROIs in the second image includes:

automatically determining a third ROI of the one or more ROIs based on a segmentation algorithm;

automatically determining a fourth ROI of the one or more ROIs based on a deformable registration result by performing a deformable registration between the first image and the second image; and

automatically determining a fifth ROI of the one or more ROIs based on a rigid registration result by performing a rigid registration between the first image and the second image,

wherein at least two of an operation for automatically determining the third ROI of the one or more ROIs based on the segmentation algorithm, an operation for automatically determining the fourth ROI of the one or more ROIs based on the deformable registration result by performing the deformable registration between the first image and the second image, and an operation for automatically determining the fifth ROI of the one or more ROIs based on the rigid registration result by performing the rigid registration between the first image and the second image are performed in parallel.

6. (Currently Amended) The method of claim 2any one of claims 2-5, wherein the determining the target radiation therapy plan includes:

displaying the first dose distribution in the second image based on the one or more ROIs;

determining a dose volume diagram (DVH) based on the first dose distribution and the one or more ROIs;

displaying the DVH;

determining, based on the estimated first dose distribution or the DVH in the second image, whether to generate the second radiation therapy plan associated with the second image or continue using the first radiation therapy plan to treat the one or more target objects;

obtaining one or more constraint conditions related to a radiation therapy to the one or more ROIs; and

generating the second radiation therapy plan based on the one or more constraint conditions and the one or more ROIs in the second image, wherein

an operation for displaying the first dose distribution in the second image based on the one or more ROIs is performed in parallel with at least one of an operation for estimating the DVH based on the first dose distribution and the one or more ROIs and an operation for displaying the DVH; or

an operation for determining, based on the estimated first dose distribution or the DVH in the second image, whether to generate the second radiation therapy plan associated with the second image or continue using the first radiation therapy plan to treat the one or more target objects is performed in parallel with at least one of an operation for obtaining the one or more constraint conditions related to the radiation therapy to the one or more ROIs and an operation for generating the second radiation therapy plan based on the one or more constraint conditions and the one or more ROIs in the second image.

7. (Original) The method of claim 1, wherein the determining the target radiation therapy plan includes:

determining one or more ROIs in the second image;

generating a first intermediate plan by updating, based on the one or more determined ROIs, the first radiation therapy plan;

updating the first intermediate plan by performing a plurality of iterations, a tth (t=1, 2, 3…) iteration of the plurality of iterations including:

receiving a second instruction for modifying or accepting a tth group of ROIs, the tth group of ROIs including at least one of the one or more determined ROIs;

processing the tth group of ROIs based on the second instruction; and

generating a (t+1)th intermediate plan based on the tth intermediate plan and the processed tth group of ROIs; and

designating the intermediate plan generated in a last iteration of the plurality of iterations as the second radiation therapy plan, wherein

an operation for generating the first intermediate plan by updating, based on the one or more determined ROIs, the first radiation therapy plan is performed in parallel with at least one of an operation for receiving the instruction for modifying or accepting the first group of ROIs and an operation for processing the first group of ROIs based on the instruction for modifying or accepting the first group of ROIs; or

an operation for generating the (t+1)th intermediate plan based on the tth intermediate plan and the processed tth group of ROIs is performed in parallel with at least one of an operation for receiving the instruction for modifying or accepting the (t+1)th group of ROIs and an operation for processing the (t+1)th group of ROIs based on the instruction for modifying or accepting the (t+1)th group of ROIs.

8. (Original) The method of claim 7, wherein the determining the target radiation therapy plan further includes:

estimating an amount of work for manually modifying each of the one or more ROIs that are automatically determined;

assigning the one or more ROIs into a plurality of groups based on the amounts of work for manually modifying the one or more ROIs; and

generating an instruction that prompts a user to modify, based on the group assignments, the one or more ROIs.

9. (Cancelled)

10. (Original) The method of claim 1, wherein at least one of an operation for obtaining the first image and an operation for obtaining the first radiation therapy plan is performed during the second scan.

11. (Original) A system for generating an adaptive radiation therapy plan, comprising:

at least one storage device storing a set of instructions; and

at least one processor in communication with the at least one storage device, wherein when the at least one processor executes the set of instructions, the system is caused to perform operations including:

obtaining a first image related to one or more target objects generated by a first scan;

obtaining a first radiation therapy plan for treating the one or more target objects, the first radiation therapy plan being associated with the first image;

obtaining a second image related to the one or more target objects generated by a second scan, the second scan being performed later than the first scan; and

determining, based on the first radiation therapy plan, the first image, and the second image, a target radiation therapy plan to treat the one or more target objects, the target radiation therapy plan being the first radiation therapy plan or a second radiation therapy plan associated with the second image, wherein at least a portion of the determining the target radiation therapy plan are performed in parallel.

12. (Original) The system of claim 11, wherein the determining the target radiation therapy plan includes:

generating a registration result by performing image registration between the first image and the second image;

determining one or more regions of interest (ROIs) in the second image based on the registration result;

determining an accumulated dose in the second image based on the registration result, the accumulated dose having been delivered to the one or more target objects; and

estimating a first dose distribution in the second image based on the first radiation therapy plan, wherein

an operation for determining the one or more ROIs in the second image based on the registration result and an operation for determining the accumulated dose in the second image based on the registration result are performed in parallel; or

an operation for estimating the first dose distribution in the second image based on the first radiation therapy plan is performed in parallel with at least one of an operation for generating the registration result by performing the image registration between the first image and the second image and the operation for determining the one or more ROIs in the second image based on the registration result.

13. (Original) The system of claim 12, wherein the determining the one or more ROIs in the second image includes:

automatically determining one or more first ROIs of the one or more ROIs in the second image;

receiving a first instruction for modifying or accepting the one or more first ROIs that are automatically determined;

processing at least one of the one or more first ROIs based on the first instruction; and

automatically determining one or more second ROIs of the one or more ROIs in the second image,

wherein an operation for automatically determining the one or more second ROIs of the one or more ROIs in the second image is performed in parallel with at least one of an operation for receiving the first instruction for modifying or accepting the one or more first ROIs that are automatically determined and an operation for processing the at least one of the one or more first ROIs based on the first instruction.

14. (Currently Amended) The system of claim 12[[ or 13]], wherein the determining the one or more ROIs in the second image includes:

estimating a length of time for determining each of the one or more ROIs;

ranking the lengths of time; and

automatically determining the one or more ROIs based on the ranking result.

15. (Currently Amended) The system of claim 12any one of claims 12-14, wherein the determining the one or more ROIs in the second image includes:

automatically determining a third ROI of the one or more ROIs based on a segmentation algorithm;

automatically determining a fourth ROI of the one or more ROIs based on a deformable registration result by performing a deformable registration between the first image and the second image; and

automatically determining a fifth ROI of the one or more ROIs based on a rigid registration result by performing a rigid registration between the first image and the second image,

wherein at least two of an operation for automatically determining the third ROI of the one or more ROIs based on the segmentation algorithm, an operation for automatically determining the fourth ROI of the one or more ROIs based on the deformable registration result by performing the deformable registration between the first image and the second image, and an operation for automatically determining the fifth ROI of the one or more ROIs based on the rigid registration result by performing the rigid registration between the first image and the second image are performed in parallel.

16. (Currently Amended) The system of claim 12any one of claims 12-15, wherein the determining the target radiation therapy plan includes:

displaying the first dose distribution in the second image based on the one or more ROIs;

determining a dose volume diagram (DVH) based on the first dose distribution and the one or more ROIs;

displaying the DVH;

determining, based on the estimated first dose distribution or the DVH in the second image, whether to generate the second radiation therapy plan associated with the second image or continue using the first radiation therapy plan to treat the one or more target objects;

obtaining one or more constraint conditions related to a radiation therapy to the one or more ROIs; and

generating the second radiation therapy plan based on the one or more constraint conditions and the one or more ROIs in the second image, wherein

an operation for displaying the first dose distribution in the second image based on the one or more ROIs is performed in parallel with at least one of an operation for estimating the DVH based on the first dose distribution and the one or more ROIs and an operation for displaying the DVH; or

an operation for determining, based on the estimated first dose distribution or the DVH in the second image, whether to generate the second radiation therapy plan associated with the second image or continue using the first radiation therapy plan to treat the one or more target objects is performed in parallel with at least one of an operation for obtaining the one or more constraint conditions related to the radiation therapy to the one or more ROIs and an operation for generating the second radiation therapy plan based on the one or more constraint conditions and the one or more ROIs in the second image.

17. (Original) The system of claim 11, wherein the determining the target radiation therapy plan includes:

determining one or more ROIs in the second image;

generating a first intermediate plan by updating, based on the one or more determined ROIs, the first radiation therapy plan;

updating the first intermediate plan by performing a plurality of iterations, a tth (t=1, 2, 3…) iteration of the plurality of iterations including:

receiving a second instruction for modifying or accepting a tth group of ROIs, the tth group of ROIs including at least one of the one or more determined ROIs;

processing the tth group of ROIs based on the second instruction; and

generating a (t+1)th intermediate plan based on the tth intermediate plan and the processed tth group of ROIs; and

designating the intermediate plan generated in a last iteration of the plurality of iterations as the second radiation therapy plan, wherein

an operation for generating the first intermediate plan by updating, based on the one or more determined ROIs, the first radiation therapy plan is performed in parallel with at least one of an operation for receiving the instruction for modifying or accepting the first group of ROIs and an operation for processing the first group of ROIs based on the instruction for modifying or accepting the first group of ROIs; or

an operation for generating the (t+1)th intermediate plan based on the tth intermediate plan and the processed tth group of ROIs is performed in parallel with at least one of an operation for receiving the instruction for modifying or accepting the (t+1)th group of ROIs and an operation for processing the (t+1)th group of ROIs based on the instruction for modifying or accepting the (t+1)th group of ROIs.

18. (Original) The system of claim 17, wherein the determining the target radiation therapy plan further includes:

estimating an amount of work for manually modifying each of the one or more ROIs that are automatically determined;

assigning the one or more ROIs into a plurality of groups based on the amounts of work for manually modifying the one or more ROIs; and

generating an instruction that prompts a user to modify, based on the group assignments, the one or more ROIs.

19. (Original) The system of claim 18, wherein the instruction prompts the user to batch process ROIs of the one or more ROIs that are assigned to a same group.

20. (Original) The system of claim 11, wherein at least one of an operation for obtaining the first image and an operation for obtaining the first radiation therapy plan is performed during the second scan.

21-30. (Cancelled)

31. (Original) A non-transitory computer readable medium, comprising at least one set of instructions for generating an adaptive radiation therapy plan, wherein when executed by one or more processors of a computing device, the at least one set of instructions causes the computing device to perform a method, the method comprising:

obtaining a first image related to one or more target objects generated by a first scan;

obtaining a first radiation therapy plan for treating the one or more target objects, the first radiation therapy plan being associated with the first image;

obtaining a second image related to the one or more target objects generated by a second scan, the second scan being performed later than the first scan; and

determining, based on the first radiation therapy plan, the first image, and the second image, a target radiation therapy plan to treat the one or more target objects, the target radiation therapy plan being the first radiation therapy plan or a second radiation therapy plan associated with the second image, wherein at least a portion of the determining the target radiation therapy plan are performed in parallel.

REMARKS

By the above amendments, Applicant amends claims 4-6 and 14-16 to remove multiple dependency.  Applicant also cancels claims 9 and 21-30 without prejudice or disclaimer of the subject matter thereof.  No new claims are added.  Therefore, no new matter has been introduced.  The entry of the amendment is respectfully requested.

Applicant submits that the present Application is in condition for examination and respectfully requests the same.  The Examiner is cordially invited to contact Applicant's representative at the number provided below to resolve any issues promptly.

|  |  | Respectfully submitted, METIS IP LLC |  |
| --- | --- | --- | --- |
| Date: | September 15, 2022 | By: | /Xiaobing Zhang/ |
|  |  |  | Xiaobing Zhang Registration No. 63,927 |
|  |  |  | Tel: 202.580.8894 ext. 102 |